# MeshSat Field Kit — GPIO Pin Allocation

Raspberry Pi 5 (Ubuntu Server 24.04) — UART2 + I2C + GPIO + DSI

parallax (RockBLOCK 9704 IMT)  •  DCF77 Receiver  •  Geekworm X1202 UPS  •  Raspberry Pi Touch Display 2 (DSI)

## Notes

Total Pi GPIO used: BCM 2, 3, 4, 5, 6, 12, 16, 20, 23, 24, 26 (11 pins) + power/ground.

DCF77 occupies separate pins — no conflicts with satellite modem or UPS.

Arrow direction: → = Pi output to device, ← = device output to Pi, ↔ = bidirectional.

All signals are 3.3V LVTTL — no level shifters needed.

UART2 device path: /dev/ttyAMA2  •  GPIO chip: /dev/gpiochip4 (RP1)

9704 baud: 230400 8N1  •  DCF77: digital pulse on GPIO 12  •  X1202: I2C at 0x36

I_BTD (9704 pin 7) is open-drain — always read with --bias=pull-up.

Touch Display 2 connects via DSI FPC ribbon cable (dedicated port, no GPIO pins consumed).

Touch Display 2 draws 5V power from GPIO header pins 2+6 (shared rail with 9704).

17 BCM GPIOs remain free: 0, 1, 7, 8, 9, 10, 11, 13, 14, 15, 17, 18, 19, 21, 22, 25, 27.

SPI0 (BCM 7/8/9/10/11) and I2C0 (BCM 0/1) fully available for future use.

UART0 (BCM 14/15) is NOT used — it causes kernel panic when 9704 TXD drives BCM 15 during boot.

## Ubuntu /boot/firmware/config.txt

[pi5]
dtoverlay=uart2-pi5
dtoverlay=vc4-kms-dsi-7inch
display_lcd_rotate=2

# Also in [all] section:
enable_uart=0
# cmdline.txt: NO console=serial0,115200

## RockBLOCK 9704 IMT (16-pin connector)

| Pi Pin | BCM | Function | Dir | 9704 Pin | 9704 Signal |
| --- | --- | --- | --- | --- | --- |
| 2 | — | 5V | → | 15 | V_IN+ (power) |
| 6 | — | GND | — | 16 | V_IN- (power ground) |
| 7 | 4 | UART2 TX | → | 14 | RXD (data to modem) |
| 9 | — | GND | — | 1 | GND (signal ground) |
| 16 | 23 | GPIO IN (pu) | ← | 7 | I_BTD (boot status, open-drain) |
| 18 | 24 | GPIO OUT | → | 6 | P_EN (charger enable, active LOW) |
| 29 | 5 | UART2 RX | ← | 13 | TXD (data from modem) |
| 37 | 26 | GPIO OUT | → | 3 | I_EN (boot control) |

## DCF77 Receiver (DCF-1060N-800)

| Pi Pin | BCM | Function | Dir | DCF77 Pin |
| --- | --- | --- | --- | --- |
| 1 | — | 3.3V | → | V (power) |
| 20 | — | GND | — | G (ground) |
| 32 | 12 | GPIO IN | ← | T (data pulse) |
| 38 | 20 | GPIO OUT (LOW) | → | P1 (enable, hold LOW) |

## Geekworm X1202 UPS (pogo pins + GPIO)

| Pi Pin | BCM | Function | Dir | X1202 Signal |
| --- | --- | --- | --- | --- |
| 3 | 2 | I2C1 SDA | ↔ | Fuel gauge MAX17040 (0x36) |
| 5 | 3 | I2C1 SCL | ↔ | Fuel gauge MAX17040 (0x36) |
| 31 | 6 | GPIO IN | ← | AC power loss detection |
| 36 | 16 | GPIO OUT (LOW) | → | Charging control (LOW=enabled) |

## Raspberry Pi Touch Display 2 (DSI)

Connection: 22-to-15 way FPC ribbon cable → Pi 5 DSI connector (dedicated port).
Power: 5V + GND via GPIO header (pins 2 + 6, shared 5V rail with 9704).
Display: 7″ IPS, 720×1280, capacitive 5-point multitouch, landscape rotation.
Software: Chromium kiosk mode → localhost:6050 (MeshSat Vue SPA).

## GPIO Summary — All 11 BCM Pins

| BCM | Pi Pin | Device | Function |
| --- | --- | --- | --- |
| 2 | 3 | X1202 | I2C SDA |
| 3 | 5 | X1202 | I2C SCL |
| 4 | 7 | 9704 | UART2 TX → RXD |
| 5 | 29 | 9704 | UART2 RX ← TXD |
| 6 | 31 | X1202 | AC power loss (input) |
| 12 | 32 | DCF77 | Data pulse (input) |
| 16 | 36 | X1202 | Charging control (output) |
| 20 | 38 | DCF77 | P1 enable (output, LOW) |
| 23 | 16 | 9704 | I_BTD boot status (input, pull-up) |
| 24 | 18 | 9704 | P_EN charger (output, active LOW) |
| 26 | 37 | 9704 | I_EN boot control (output) |

MeshSat Field Kit — MESHSAT-403  •  Generated 2026-04-15  •  Rev D (UART2 replaces UART0, X1202 UPS added)

Sources: RP1 Peripherals Datasheet Table 4 (raspberrypi.com)  •  Ground Control RockBLOCK 9704 Hardware (docs.groundcontrol.com)  •  Geekworm X1202 Wiki + suptronics/x120x (github.com)  •  Raspberry Pi Touch Display 2 product brief (raspberrypi.com)