MeshSat Field Kit — GPIO Pin Allocation

Raspberry Pi 5 (Ubuntu Server 24.04) — UART0 + DSI + GPIO

tesseract (RockBLOCK 9603 SBD)  •  parallax (RockBLOCK 9704 IMT)  •  DCF77 Receiver  •  Raspberry Pi Touch Display 2 (DSI)

Notes

Total Pi GPIO used: BCM 12, 14, 15, 16, 20, 23, 24 (7 pins) + power/ground.

Satellite modem columns are mutually exclusive — one modem per kit.

DCF77 occupies separate pins — no conflicts with either satellite modem.

Arrow direction: → = Pi output to device, ← = device output to Pi.

All signals are 3.3V LVTTL — no level shifters needed.

UART0 device path: /dev/ttyAMA0  •  GPIO chip: /dev/gpiochip4 (RP1)

9603 baud: 19200 8N1  •  9704 baud: 230400 8N1  •  DCF77: digital pulse on GPIO 12

Touch Display 2 connects via DSI FPC ribbon cable (dedicated port, no GPIO pins consumed).

Touch Display 2 draws 5V power from GPIO header pins 2+6 (shared rail with satellite modem).

Touch Display 2: 720×1280, landscape rotation via display_lcd_rotate=2 in config.txt.

19 BCM GPIOs remain free. SPI0 (BCM 8/9/10/11) and I2C1 (BCM 2/3) fully available for future use.

Rev F 9603 boards: use 2N7000 MOSFET on ON/OFF (pin 7) — do not drive directly from GPIO.

DCF77 pin P1 must be held LOW for the module to operate. Alternative: tie P1 directly to GND.

Ubuntu /boot/firmware/config.txt

[pi5]

dtoverlay=uart0-pi5

dtoverlay=vc4-kms-dsi-7inch

display_lcd_rotate=2

| Pi Pin | BCM | Function | tesseract (9603) | parallax (9704) | DCF77 |
| --- | --- | --- | --- | --- | --- |
| 1 | — | 3.3V | — | — | VCC (pin V) |
| 2 | — | 5V | 5V In (pin 8) | V_IN+ (pin 15) | — |
| 6 | — | GND | GND (pin 10) | GND (pin 16) | — |
| 8 | GPIO 14 | UART0 TX | → TXD (pin 6) | → RXD (pin 14) | — |
| 9 | — | GND | — | GND (pin 1) | — |
| 10 | GPIO 15 | UART0 RX | ← RXD (pin 1) | ← TXD (pin 13) | — |
| 16 | GPIO 23 | GPIO IN | ← RI (pin 5) | ← I_BTD (pin 7) | — |
| 18 | GPIO 24 | GPIO OUT | → ON/OFF (pin 7) | → P_EN (pin 6) | — |
| 20 | — | GND | — | — | GND (pin G) |
| 32 | GPIO 12 | GPIO IN | — | — | ← Data (pin T) |
| 36 | GPIO 16 | GPIO | ← NetAv (pin 4) | → I_EN (pin 3) | — |
| 38 | GPIO 20 | GPIO OUT | — | — | → P1 (hold LOW) |

Touch Display 2 (not in GPIO table — uses DSI port)

Connection: 22-to-15 way FPC ribbon cable → Pi 5 DSI connector (dedicated port).

Power: 5V + GND via GPIO connector cable (pins 2 + 6, shared 5V rail).

Display: 7″ IPS, 720×1280, capacitive 5-point multitouch, anti-glare, ±85° viewing angle.

Active area: 154.56 × 86.94 mm  •  Module: 189.32 × 120.24 × 8.55 mm  •  Weight: 244g.

Mount: landscape on 200×300×3mm polystyrene plate (5mm side clearance), on top of equipment.

Software: Chromium kiosk mode → localhost:6050 (MeshSat Vue SPA).

MeshSat Field Kit — MESHSAT-403  •  Generated 2026-04-14  •  Rev C (touchscreen replaces e-paper)

Sources: RP1 Peripherals Datasheet (raspberrypi.com)  •  Ground Control docs (docs.groundcontrol.com)  •  RockBLOCK 9704 C Library (github.com/rock7)  •  Raspberry Pi Touch Display 2 product brief (raspberrypi.com)